Aerodynamic shape factor

In accordance with AS 1170.2:2021

Tedds calculation version 1.0.07

Library item: Heading

Input details

Type of structure;	Attached canopies, awnings and carports

Selected structure;	Attached canopies and awning

Proportion of c/s area under roof blocked;	Ablocked = 0.00 %

Avg. roof ht of structure the canopy is attached to;	h = 5.00 m

Height from ground to the attached canopy;	hc = 3.00 m

Width of canopy, awning, carport from free face bld.;	wc = 2.10 m

Length of canopy / awning parallel to attached bldg.;	b = 4.50 m

Area reduction factor (cl. B.1.2)

When q is equal to 0o;	Ka_0 = 1

Tributary area contributing to force when q is 90o;	At = 10.00 m2

Area reduction factor (table 5.4);	Ka_90 = if(At <= 10 m2, 1, if(At < 25 m2, (1 – (0.1  (At – 10 m2)) / 15 m2 ), if(At < 100 m2, (0.9 – (0.1  (At – 25 m2)) / 75 m2), 0.8))) = 1.00

Area reduction factor when q is 90o (table 5.4);	Ka_90 = 1.00

Cladding tributary area contributing to force;	At_cladding = 10.00 m2

Area reduction factor (table 5.4);	Ka_cladding = if(At_cladding <= 10 m2, 1, if(At_cladding < 25 m2, (1 – (0.1  (At_cladding – 10 m2)) / 15 m2 ), if(At_cladding < 100 m2, (0.9 – (0.1  (At_cladding – 25 m2)) / 75 m2), 0.8))) = 1.00

Cladding area reduction factor for q at 90o (T5.4);	Ka_cladding = 1.00

Local net pressure factor (cl. B.1.3)

Shortest horz plan dimn of free roof or canopy;	w = min (b,wc) = 2.10 m

a = 20  w / 100 = 0.42 m

Angle of slope of a roof;	a = 0

When q is equal to 0o;	Kl = 1

When q is equal to 90o or 270o Case 1;	Kl_cladding1 = 1.5

When q is equal to 90o or 270o Case 2;	Kl_cladding2 = 2

When q is equal to 90o or 270o Case 3;	Kl_cladding = 3

Library item: Input for attached canopies

Net pressure coefficient (cl. B.4.1)

hc_upon_h;	hc_upon_h = hc / h = 0.60

Pos. net pressure coefficient (table B.9);	Cpn_pos = if (hc / h <= 0.1, 1.2, if (hc / h == 0.2,0.7, if (hc / h == 0.5, 0.5, if (hc / h == 0.75, 0.4, if (hc / h == 1, 0.2, if (and((hc / h > 0.1),(hc / h < 0.2)), (1.2 – (0.5  ((hc / h) - 0.1)/0.1)), if (and((hc / h > 0.2),( hc / h < 0.5)), (0.7 – (0.3  ((hc / h) - 0.2)/0.3)), if (and((hc / h >0.5), (hc / h < 0.75)), (0.5 – (0.1  ((hc / h) - 0.5)/0.25)), (0.4 – (0.2  ((hc / h) - 0.75)/0.25)))))))))) = 0.46

Pos. net pressure coefficient (table B.9);	Cpn_pos = 0.46

Cpn_0.75 = max(((- 0.3) - (0.2  (hc / wc))), -1.5) = -0.586;

Cpn_1 = max(((- 0.3) - (0.6  (hc / wc))), -1.5) = -1.157;

Neg. net pressure coefficient;	Cpn_neg = if (hc / h< 0.5, -0.2, if (hc / h == 0.5, -0.3, if (and((hc / h > 0.5), (hc / h < 0.75)), (-0.3 + ((Cpn_0.75 + 0.3)  (hc_upon_h - 0.5) / 0.25)), if (hc / h == 0.75, Cpn_0.75, if(and ((hc / h > 0.75), (hc / h < 1)), (Cpn_0.75 + ((Cpn_1 - Cpn_0.75)  (hc_upon_h - 0.75) / 0.25)), Cpn_1))))) = -0.41

Neg. net pressure coefficient (table B.9);	Cpn_neg = -0.41

Library item: C_{p.n} attach canopies theta 0

Aerodynamic shape factor for normal net pressure (cl. B.4.1)

Positive value;	Cshp_pos = Ka_0  Kl  Cpn_pos = 0.46

Negative value;	Cshp_neg = Ka_0  Kl  Cpn_neg = -0.41

Library item: Factor attached canopies normal

Calculation of net pressure coefficient when when q = 90° (cl. B.3.1, table (B.4(B)))

x1 = min(h, b) = 4.50 m; x2 = min(2  h, b) = 4.50 m;

Zone 1;	<= 4.50 m from the windward edge

Neg. pressure coefficient;	Cpn_neg1 = if (Ablocked <= 0.50, -0.3, if (Ablocked < 0.75, (-0.3 + ((-0.7)  (Ablocked – 0.50) / 0.25)), -1.0)) = -0.30

Neg. pressure coefficient;	Cpn_neg1 = -0.30

Pos. pressure coefficient;	Cpn_pos1 = 0.4

Library item: Coefficient for attached canopies theta 90, zone 1

Aerodynamic shape factor for normal net pressure when q = 90° (cl. B.3.1)

Zone 1;	<= 4.50 m from the windward edge

Negative first zone;	Cshp_neg1 = Ka_90  Kl  Cpn_neg1 = -0.30

Positive first zone;	Cshp_pos1 = Ka_90  Kl  Cpn_pos1 = 0.40

For the design of cladding and immediate cladding support (cl. B.3.1)

Area = 0.25  a  a = 0.04 m2; a1 = 0.5  a = 0.21 m

Case 1 - Pressures on an area between 0 and 1.0a2;WARNING: field has not been calculated<Error: undefined variable 'Area_1'>;within a distance 1.0a;(0.42 m);from an upwind roof edge, or downwind of a ridge with a pitch of 10deg or more.

Negative cladding first zone;	Cshp_neg1_c1 = Ka_cladding  Kl_cladding1  Cpn_neg1 = -0.45

Positive cladding first zone;	Cshp_pos1_c1 = Ka_cladding  Kl_cladding1  Cpn_pos1 = 0.60

Case 2 - Pressures on an area of 0.25a2;(0.04 m2);or less, within a distance 0.5a;(0.21 m);from an upwind roof edge, or downwind of a ridge with a pitch of 10deg or more

Negative cladding first zone;	Cshp_neg1_c2 = Ka_cladding  Kl_cladding2  Cpn_neg1 = -0.60

Positive cladding first zone;	Cshp_pos1_c2 = Ka_cladding  Kl_cladding2  Cpn_pos1 = 0.80

Case 3 - Upward net pressures on an area of 0.25a2;(0.04 m2);or less, within a distance 0.5a;(0.21 m);from an upwind corner of a free roof with a pitch of less than 10deg.

Negative cladding first zone;	Cshp_neg1_c = Ka_cladding  Kl_cladding  Cpn_neg1 = -0.90

Library item: Factor for attached canopies theta 90, zone 1

Aerodynamic shape factor for frictional drag

Surface description;	With ribs across wind dir

Table = ".2:2002 Table B.3";AS1170.2:2002 Table D3

Frictional drag force coefficient (table B.3);	Cf = 0.04

Aerodynamic shape factor for frictional drag;	Cshp_drag = Cf = 0.04

Library item: Factor attached canopies drag

Wind loading

In accordance with AS 1170.2:2021

Tedds calculation version 1.0.07

Library item: Heading

Regional wind speeds (cl. 3.2)

Regional wind speeds (VR) for all directions based on peak gust wind data shall be as given in Table 3.1 AS/NZS 1170.2 for the regions shown in Figure 3.1(A) and Figure 3.1(B) where R (average recurrence interval) is the inverse of the annual probability of exceedance of the wind speed for ultimate or serviceability limit states.  Refer to AS/NZS 1170.0 for information on values of annual probability of exceedance appropriate.

Inverse of annual probability of wind speed exceedance (serviceability);

Rservice = 20

Inverse of annual probability of wind speed exceedance (ultimate);

Rult = 500

Wind region (Figures 3.1(A) & 3.1(B));	A5

Regional wind speed (serviceability);	VR.service = Round(67m/s - 41m/s ´ Rservice-0.1, 0) = 37 m/s

Regional wind speed (ultimate);	VR.ult = Round(67m/s - 41m/s ´ Rult-0.1, 0) = 45 m/s

Multipliers

Wind direction multiplier (cl. 3.3);	Md = 1.00

Climate change multiplier (Table 3.3);	Mc = 1.00

Library item – wdm.b

Terrain/height multiplier (cl. 4.2)

Terrain category;	TerrainCat = 3.00 ;

Height;	z = 3.00 m

Specified terrain/height multiplier;	Mzcat = 0.83

Library item – thm.specified

Shielding multiplier (cl. 4.3)

Shielding multiplier (cl. 4.3);	Ms = 1.00

Library item – sm.2

Topographic multiplier (cl. 4.4);	Mt = 1.00

Library item – tm.2

Wind speed

Site wind speed (serviceability) (cl. 2.2);	Vsit.bs = VR.service  Md  Mc  (Mzcat  Ms  Mt) = 30.71 m/s

Site wind speed (ultimate) (cl. 2.2);	Vsit.bu = VR.ult  Md  Mc  (Mzcat  Ms  Mt) = 37.35 m/s

The site wind speed is independent of the type or shape of structure.

The design wind speed (Vdes.q) equals the maximum value of site wind speed in the range q = b +/- 45° where b is the cardinal direction clockwise from true North and q is the angle to the building orthogonal axes.  The design wind speed considers directional effects and variations with height.

Design wind speed (serviceability) (cl. 2.3);	Vdes.qs = Vsit.bs ;

Design wind speed (ultimate) (cl. 2.3);	Vdes.qu = max(Vsit.bu , 30 m/s) = 37.35 m/s;  in this case.

Library item – dws.b

Design wind pressure (cl. 2.4)

External aerodynamic shape factor;	Cshp,e = 1.00

Internal aerodynamic shape factor;	Cshp,i = 1.00

Dynamic response factor;	Cdyn = 1.00

rair = 1.2 kg/m3

External design wind pressure (serviceability);	pqs,e = 0.5  rair  Vdes.qs2  Cshp,e  Cdyn = 0.57 kPa

External design wind pressure (ultimate);	pqu,e = 0.5  rair  Vdes.qu2  Cshp,e  Cdyn = 0.84 kPa

Internal design wind pressure (serviceability);	pqs,i = 0.5  rair  Vdes.qs2  Cshp,i  Cdyn = 0.57 kPa

Internal design wind pressure (ultimate);	pqu,i = 0.5  rair  Vdes.qu2  Cshp,i  Cdyn = 0.84 kPa

Resultant design wind pressure (serviceability);	pqs = pqs,e - pqs,i = 0.00 kPa

Resultant design wind pressure (ultimate);	pqu = pqu,e - pqu,i = 0.00 kPa

Library item – wp.b